GLASS FIBER ACOUSTICAL
INTERIOR PARTITION INSULATION

# Guide Specification SECTION 09 81 16

# Acoustic Blanket Insulation

SPECIFIER'S RESPONSIBILITY: This is a general specification guide, intended to be used by experienced construction

professionals, in conjunction with good construction practice and professional judgment. This guide is to aid in the creation of a

complete building specification that is to be fully reviewed and edited by the engineer. Sections of this guide should be included,

edited, or omitted based on the requirements of a specific project. It is the responsibility of both the specifier and the purchaser to

determine if a product or system is suitable for its intended use. Neither Owens Corning, nor any of its subsidiary or affiliated

companies, assume any responsibility for the content of this specification guide relative to actual projects and specifically disclaim

any and all liability for any errors or omissions in design, detail, structural capability, attachment details, shop drawings, or other construction-related details, whether based upon the information provided by Owens Corning or otherwise.

SECTION 09 81 16

ACOUSTIC BLANKET INSULATION

PART 1 — GENERAL

  - SUMMARY

- Section Includes: Glass fiber acoustical blanket insulation for interior partitions.

- Related Sections:

  - Section 07 21 00 - Mineral Wool Insulation.

  - Section 07 21 16 - Glass Fiber Batt Insulation.

  - Section 07 21 18 - Glass Fiber Blanket Insulation.

  - SUBMITTALS

- Product Data: Submit product characteristics, performance criteria, and limitations, including installation instructions.

- Sustainable Design: Submit manufacturer’s sustainable design certifications as specified with each product.

  - QUALITY ASSURANCE

- Mock-Up: If requested, provide a mock-up of materials proposed for use for review of workmanship. Accepted mock-ups may remain in place.

- Preconstruction Meeting: Convene a minimum of two weeks prior to commencing work of this section. Agenda shall include materials proposed for use, sequence of construction and coordination with installation of adjacent and covering materials.

  - DELIVERY, STORAGE, AND HANDLING

- Delivery: Deliver materials to the job site in original packages, containers, or bundles bearing the brand name and manufacturer’s identification.

- Storage: Store materials in dry locations with adequate ventilation, free from water, and in such a manner to permit easy access for inspection and handling.

- Handling: Handle using procedures recommended by the manufacturer for materials and personnel.

  - WARRANTY

A.	Warranty: Provide manufacturer’s standard limited warranty against manufacturing defects.

PART 2 — PRODUCTS

  - MANUFACTURER

- Basis-of-Design Manufacturer: Owens Corning Insulating Systems, LLC, Toledo, OH 43659; www.owenscorning.com.

  - ACOUSTIC BLANKET INSULATION (SOUND ATTENUATION BATTS), FIRE-RATED (Metal Framing)

- Type: Unfaced glass-fiber acoustical insulation, complying with ASTM C665, Type I.

  - Thickness:  3 1/2 inches (89 mm).

  - Thickness:  2 1/2 inches (64 mm).

  - Width:  16 to 24 inches (406 to 609 mm).

  - Length:  96 inches (2438 mm).

- Surface Burning Characteristics: ASTM E84.

  - Maximum flame spread: <25

  - Maximum smoke developed: <50

- Combustion Characteristics: Passes ASTM E136.

- Fire Resistance Ratings: Part of ASTM E119 fire tested wall assemblies.

- Sound Transmission Class: ASTM C423, STC based on manufacturer's published data on thickness and wall assembly.

- Dimensional Stability: Linear Shrinkage less than 0.1%

  - ACOUSTIC BLANKET INSULATION (ACOUSTIC BATTS), FIRE-RATED (Wood Framing)

- Type: Unfaced glass fiber acoustical insulation, complying with ASTM C 665, Type I.

  - Thickness:  3 1/2 inches (89 mm).

  - Width:  15 1/2 to 23 1/2 inches (mm).

  - Length:  96 inches (2438 mm).

- Surface Burning Characteristics: ASTM E84.

  - Maximum flame spread: <25.

  - Maximum smoke developed: <50

- Fire Resistance Ratings: Part of ASTM E119 fire tested wall assemblies.

- Sound Transmission Class: ASTM C423, STC ___

- Dimensional Stability: Linear Shrinkage less than 0.1%

PART 3 — EXECUTION

  - EXAMINATION

- Examine the areas and conditions under which work of this section will be installed. Verify that adjacent materials are dry and ready to receive insulation. Verify mechanical and electrical services within walls have been tested and inspected.

- Provide written report listing conditions detrimental to performance of work in this section. Do not proceed with installation until unsatisfactory conditions have been corrected.

  - INSTALLATION

- Comply with manufacturer's installation instructions.  Do not use unfaced insulation in exposed applications where there is potential for skin contact and irritation.  Friction-fit blanket insulation in place, until the interior finish is applied. Install batts to fill entire stud cavity, with no gaps, voids, or areas of compression. If stud cavity is less than 8 feet in height, cut lengths to friction fit against floor and ceiling tracks. Walls with penetrations require that insulation be carefully cut to fit around outlets, junction boxes, and other irregularities.

- Where walls are not finished on both sides or where insulation does not fill the cavity depth, install supplementary support to hold product in place.

- Where insulation must extend higher than 8 feet, provide temporary support to hold product in place, until finish material is applied.

  - PROTECTION

- Protect installed insulation from damage due to weather and physical abuse until protected by permanent construction.

Pub. No. 10019379-A. Printed in U.S.A. May 2020.

© 2020 Owens Corning. All Rights Reserved.

End of Section 09 81 16

Pub. No. 10024959. June 2021.

The color PINK is a registered trademark of Owens Corning. All Rights Reserved.

© 2021 Owens Corning. All Rights Reserved.